This guide will walk your company through installing the development environment, connecting to the hardware, and transferring the firmware to the Raspberry Pi Pico 2WYou can watch similar steps on YouTube: (https://youtu.be/V5yQ2-BPWLw?si=BpnSXxz7sOUPejI9)

#### 1. Download & Install Thonny IDE

Before connecting your hardware, you must install Thonny on your computer.

- For Windows: * Go to thonny.org and download the Windows Installer (64-bit).

  - Run the .exe file. If a "Windows protected your PC" message appears, click More info → Run anyway.

- For macOS: * Go to thonny.org and download the .pkg file.

  - If blocked by security settings, go to System Settings > Privacy & Security and click Open Anyway.

#### 2. Initializing Thonny (First-Time Setup)

Configure Thonny to communicate with the MicroPython language on your console.

- Connect the Console: Plug your Raspberry Pi Pico 2W into the computer using a micro-USB data cable.

- Enable the Files Sidebar: In the top menu, go to View → Files.

- Select the Interpreter: Click the text in the bottom-right corner (likely "Python 3.x") and select MicroPython (Raspberry Pi Pico) from the list.

- The "Handshake" (Reset): Click the Red Stop Button in the top toolbar.

  - The Shell at the bottom should display: MicroPython v1.2x.x... on Raspberry Pi Pico.

  - The Files pane will now split into "This computer" (top) and "Raspberry Pi Pico" (bottom).

#### 3. Uploading the Firmware

Move the project files from your company's computer to the console's memory.

- Locate the Files: In the top section (This computer), navigate to your /Software folder.

- Select the Firmware: Highlight main.py, diagnostics.py, ili9341.py, and boot.py.

- Transfer: Right-click the highlighted files and select "Upload to /". They will appear in the bottom section (Raspberry Pi Pico).

#### 4. Functional Testing & Switching Scripts

The Pico can only run one program at a time. Follow this workflow to test your build.

- Run Diagnostics: Double-click diagnostics.py in the bottom section and click the Green Play Button.

- Test Hardware: Verify the screen, joystick, and buzzer are responsive.

- Switch to Main Game:

  - STOP: You must click the Red Stop Button first to end the diagnostic script.

  - OPEN: Click the tab for main.py at the top of the editor.

  - PLAY: Click the Green Play Button to launch the main console interface.

#### 5. Troubleshooting for Companies

- "Device is Busy": A script is currently running. Click the Red Stop Button before trying to upload or run new files.

- Pico Not Found: Ensure you are using a Data Cable. Some cables are "power-only" and cannot transfer code.

- Missing "Upload to /": Ensure you have right-clicked the files in the top (Computer) section of the pane.

Next Step: Proceed to Guide 04: Console_User_Manual to connect the battery and complete the build!